Project506

Water Pump:

Components Required:

- DC Motor: The motor to control.

- Relay Module

- External Power Supply: To power the DC motor

- Flyback Diode: Across the motor terminals to protect against voltage spikes.

Relay Module Connections:

- VCC: Connect to 5V of the Raspberry Pi.

- GND: Connect to the Raspberry Pi's GND.

- IN: Connect to a GPIO pin of the Raspberry Pi (GPIO17).

Relay to Motor Connections:

- NO (Normally Open): Connect to one terminal of the motor.

- COM (Common): Connect to the positive terminal of the power supply.

- Connect the negative terminal of the power supply to one terminal of the motor.

Flyback Diode:

- Connect across the motor terminals with the diode's cathode (the stripe) to the positive terminal.

Water levels:

Components Required:

- HC-SR04 Ultrasonic Sensor

Wiring the HC-SR04 to the Raspberry Pi:

- VCC of the HC-SR04 to 5V on the Raspberry Pi

- GND of the HC-SR04 to GND on the Raspberry Pi

- Trigger (TRIG) to a GPIO pin (GPIO22)

- Echo (ECHO) to another GPIO pin (GPIO27)

Hydration valve:

Components Required:

1.DC Motor: The motor to control.

2.Relay Module

3.External Power Supply: To power the DC motor

4.Flyback Diode: Across the motor terminals to protect against voltage spikes.

Relay Module Connections:

- VCC: Connect to 5V of the Raspberry Pi.

- GND: Connect to the Raspberry Pi's GND.

- IN: Connect to a GPIO pin of the Raspberry Pi (GPIO16).

Relay to Motor Connections:

- NO (Normally Open): Connect to one terminal of the motor.

- COM (Common): Connect to the positive terminal of the power supply.

- Connect the negative terminal of the power supply to one terminal of the motor.

Flyback Diode:

- Connect across the motor terminals with the diode's cathode (the stripe) to the positive terminal.

Temperate and Humidity Sensors:

Components Required:

- DHT21/AM2301 temperature and humidity sensor

- 10kΩ resistor (for the pull-up resistor)

1. Wiring the DHT21/AM2301 to Raspberry Pi:

The DHT21 sensor has three pins:

- VCC connect to 3.3V

- GND (Ground, connect to GND on the Raspberry Pi)

- DATA (Data, connected to a GPIO pin) (GPIO05)

We ’ll also need a pull-up resistor (typically 10kΩ) between the DATA and VCC lines to ensure stable communication.

Day\Night Modes:

1. Wiring the LDR sensor

We’ll need to wire your LDR sensor to the Raspberry Pi using a voltage divider circuit. Connect:

- One end of the LDR to 3.3V.

- The other end to GPIO pin (GPIO06) and also connect a resistor (10kΩ) to ground (GND).

Components;

Water Pump\ Hydration valve:

DC Motor                     Relay                                   Diode

Water levels:

Ultrasonic Sensor

Temperate and Humidity Sensors:

DHT21/AM2301                           Resistor 10k

Day\Night Modes:

LDR                                              Resistor 10k